LOT 1 Snihurivka Vocational Lyceum

The works are planned in two buildings of the complex: a dormitory with administrative premises and an educational building.

Within these premises, it is planned to carry out all the works specified in the list of works, namely: dismantling works, installation of 4 external doors, 95 internal wooden doors, 16 PVC doors and 128 windows, including installation of slopes for doors and windows. Roof repair; repair or replacement of wall plaster; floor repair; repair of the lighting system with the replacement of power distribution panels, sockets and switches; repair of water supply and sewage networks, for example, replacement of pipes, toilets and sinks. A list of complex works and their number can be found in Annex C1.

Dormitory

The dormitory is a five-story brick building with a basement (under a part of the building) and a technical underground.

On the first floor are located administrative, economic, educational and technical premises. ​On the second - residential rooms. The third, fourth and fifth floors, which by typology intended for residence students, are not exploited.

Basement

|  | Dismantling work, installation of a concrete floor from a self-leveling mixture, plastering , installation of an autonomous lighting system. |
| --- | --- |
|  | Supply and installation of a metal doors with slopes and all accessories |
|  | Supply and installation of a metal-plastic doors PVC with slopes and all accessories |
|  | Arrangement of an additional evacuation exit from the basement to the street |
|  | Supply of materials and services for the installation and connection of electric LED lamps |
|  | Supply and installation of electrical sockets and switches |
|  | Supply and installation of electrical panel |

1st floor

|  | Partial repair of plaster and flooring, painting of the walls and ceilings |
| --- | --- |
|  | Supply and installation of a metal doors with slopes and all accessories |
|  | Supply and installation of a metal-plastic doors PVC with slopes and all accessories |
|  | Dismantling of existing windows. Supply and installation of a metal-plastic window PVC with slopes and all accessories |

2nd floor

|  | Partial repair of masonry, plastering, painting, floor repairs with self-leveling screed and linoleum |
| --- | --- |
|  | Partial repair of masonry, plastering, painting, partial finishing of walls with ceramic tiles, floor repairs with self-leveling screed and ceramic tiles |
|  | Supply and installation of wooden doors with slopes and all accessories |
|  | Supply and installation of a metal-plastic doors PVC with slopes and all accessories |
|  | Dismantling of existing windows. Supply and installation of a metal-plastic window PVC with slopes and all accessories |
|  | Supply of materials and services for the installation and connection of electric LED lamps |
|  | Supply and installation of electrical sockets and switches |
|  | Supply and installation of electrical panel |

3rd, 4th and 5th floors

Closing window openings with OSB plates

Roof

Dismantling of damaged roof structures, supply and installation of asbestos-free slate sheets.

Educational building

The educational building is the only survived educational building in the lyceum complex. A two-story brick building without a basement.

The following works are planned:

1st floor

|  | Complete dismantling and installation of new floors (including insulation, screed, finishing with linoleum); partial repairs of masonry, plastering and painting (walls) |
| --- | --- |
|  | Complete dismantling and installation of new floors (including insulation, screed, finishing with ceramic tiles); partial repairs of masonry, plastering and painting (walls) |
|  | Complete dismantling and installation of new floors (including insulation, screed, finishing with ceramic tiles); partial repairs of masonry, plastering and and finishing with painting and partially with ceramic tiles (walls) |
|  | Supply and installation of wooden doors with slopes and all accessories |
|  | Supply and installation of a metal-plastic doors PVC with slopes and all accessories |
|  | Dismantling of existing windows. Supply and installation of a metal-plastic window PVC with slopes and all accessories |

2nd floor

|  | Dismantling and installation of new floors (self-leveling screed, finishing with linoleum); partial repairs of masonry, plastering and painting (walls) |
| --- | --- |
|  | Dismantling and installation of new floors (self-leveling screed, finishing with ceramic tiles); partial repairs of masonry, plastering and painting (walls) |
|  | Dismantling and installation of new floors (self-leveling screed, finishing with ceramic tiles); partial repairs of masonry, plastering and and finishing with painting and partially with ceramic tiles (walls) |
|  | Supply and installation of wooden doors with slopes and all accessories |
|  | Supply and installation of a metal-plastic doors PVC with slopes and all accessories |
|  | Dismantling of existing windows. Supply and installation of a metal-plastic window PVC with slopes and all accessories |