LOT 4 Mykolaiv Construction College.

Work is planned in two buildings: educational building #2 and workshop building #3.

Educational building #2.

The educational building is a three-story building with a basement under the entire building and an attic. The building is frameless . The walls are made of brick and shell stone.

Precast reinforced concrete floors - T-shaped beams with small reinforced concrete slabs between them.

The following works are expected to be performed:

Within the basement: dismantling work, installation of a concrete floor including self-leveling mixture, installation of doors (13 pcs), installation of door slopes, plastering, installation of a lift for people with reduced mobility, installation of a lighting system including distribution panels of the electrical network, sockets and switches, etc., laying of partitions.

Within the 1st, 2nd, 3rd floors: dismantling works, installation of windows (155 pcs) and window slopes, installation of wooden doors (37 pcs), PVC doors (10 pcs), installation of door slopes, plastering (around the windows), cladding walls and floors with ceramic tiles, plastering of walls and ceilings, installation of sinks and toilets, installation of sewer pipes and water supply.

Basement

1st floor

2nd floor

3rd floor

|  | Dismantling of existing windows. Supply and installation of a metal-plastic window PVC with slopes and all accessories |
| --- | --- |
|  | Dismantling of existing internal doors. Supply and installation of a wooden doors with slopes and all accessories |
|  | Dismantling of existing external doors. Supply and installation of a metal-plastic doors PVC with slopes and all accessories |
|  | Installation of ceramic tiles, including the installation of skirting boards made of the same material. Electrical works. Installation and connection of sanitary appliances. Installation of water and sewage pipelines. |
|  | Installation of the floor using a self-leveling mixture. Supply and applying of plaster, putty, primer and finishing paint. Electrical works |
|  | Supply of materials and services for the installation and connection of electric LED lamps |
|  | Supply and installation of electrical panel |
|  | Supply and installation of electrical sockets and switches |

Workshop building #3.

The Workshop is a four-story building without a basement with a combined coating. The building is made in a full frame. Columns, crossbars and floor slabs are prefabricated reinforced concrete. Walls are hinged prefabricated reinforced concrete panels made of lightweight concrete and brickwork. Column cell 6.0x9.0 m. The total height of the building is 16.07 m.

All the works specified in the list of works are planned to be performed within these premises, namely: dismantling works, installation of 4 internal wooden doors, and 81 windows, including construction of slopes for doors and windows. The list of comprehensive works and quantities can be found in Annex C1. Technical Bid.

1st floor

2nd floor

3rd floor

4th floor

|  | Dismantling of existing windows. Supply and installation of a metal-plastic window PVC with slopes and all accessories |
| --- | --- |
|  | Dismantling of existing internal doors. Supply and installation of a wooden doors with slopes and all accessories |